# Engine Lubrication System

# General Description

## Engine Lubrication Description

AENS0M0A1501001 (03(01)

The oil pump is of a parachoid type, installed in the timing chain cover and is driven by the crankshaft.

Oil is drawn up through the oil pump strainer and passes through the pump to the oil filter. The filtered oil flows into the main oil gallery in the cylinder block. Oil from the main gallery is supplied to the crankshaft main bearings and cylinder head. An oil passage is connected to the sub gallery via backside of a crankshaft main bearing. The following devices are lubricated or pressurized by delivered oil as follows.

- Crankshaft journal

Main bearings supporting crankshaft journals are lubricated and the oil is supplied to the connecting rod bearings by means of intersecting passages drilled in the crankshaft, and then splashed from the big end of each connecting rod to lubricate cylinder wall, piston pin, piston and piston rings.

- Timing chain oil jet

Oil from an oil passage between the main gallery and main bearing is supplied to the jet and injected for lubrication of the timing chain, timing sprockets, timing chain tensioner and timing chain guide.

- Piston cooling jet

Oil from the sub gallery is injected through the jet to each piston for additional piston cooling.

- Tensioner adjuster

Oil from an oil passage connected to the main bearing provides pressure to the tensioner adjuster for adjustment of the chain tension.

- Cylinder head

Oil from the main gallery is supplied to OCVs through the timing chain cover and the rest of oil goes up to camshaft front lower housing and then divided into passages drilled in the cylinder head at intake side and exhaust side independently.

  - Intake and exhaust VVT actuators

Oil is supplied through the timing chain cover, each OCV, cylinder head, camshaft front lower housing, camshaft front upper housing and each camshaft for the camshaft timing control.

  - Camshaft journal

Oil is supplied through the passages in the cylinder head at intake side and exhaust side except No.1. No.1 journals are lubricated by oil from each OCV through passages in camshaft front lower housing and camshaft front upper housing.

  - Hydraulic lash adjuster

Through the same passages for the camshaft journals, oil is supplied. From top of each hydraulic lash adjuster, oil is injected to the valve rocker arm according to the stroke.

  - Valve rocker arm and cam surface

Splashed oil from top of the hydraulic lash adjuster through the valve rocker arm lubricates the valve rocker arm including its roller bearing and corresponding cam surface.

The oil pump is equipped with an oil relief valve. This valve relieves oil pressure when the pressure exceeds specified value.

The oil filter provides a relief valve to bypass oil when oil pressure exceeds the filter’s specification caused by clogging of the filter element due to poor maintenance (without changing the filter as scheduled). In this situation, dusty oil will circulate in the engine without filtration and causes the engine damage.

Cylinder block is provided with an oil temperature sensor for idle speed control. For the idle speed control and sensor inspection, refer to “Oil Temperature Sensor On-Vehicle Inspection” in Section 1C or “Oil Temperature Sensor Inspection” in Section 1C.